Miame Checklist

Part 1 Experiment description

-      Human hypopharyngeal cancer cell, FaDu

-        experimental variables (parent cell, FaDu vs. cisplatin-resistant cell, FaDu/DDP4)

-        n-count for each group is 1

Part 2 Array design.

-        Agilent SurePrint G3 Microarray – Human, SBC Human (4*180K) ceRNA Microarray and Agilent Human (8*60K) miRNA Microarray

-         Array type (human, ceRNA and miRNA)

-        18853 probes for mRNA, 77103 probes for lncRNA, 88371 probes for circRNA, 2549 probes for miRNA

-         Each probe for mRNA, lncRNA and circRNA was 60 mer long

Part 3 Samples

-         Cy3 labels for cells

-         There are dye controls and quality control probes in the microarray.

-        Total RNA was labeled by Low Input Quick Amp Labeling Kit, One-Color (Cat.# 5190-2305, Agilent technologies, Santa Clara, CA, US), following the manufacturer’s instructions.

-        Total RNA was extracted and purified using miRNeasy Mini Kit（Cat#217004，QIAGEN, GmBH, Germany) following the manufacturer’s instructions and checked for a RIN number to inspect RNA integration by an Agilent Bioanalyzer 2100 (Agilent technologies, Santa Clara, CA, US).

Part 4 Hybridizations

-        Each slide was hybridized with 1.65μg Cy3-labeled cRNA using Gene Expression Hybridization Kit (Cat.# 5188-5242, Agilent technologies, Santa Clara, CA, US) in Hybridization Oven (Cat.# G2545A, Agilent technologies, Santa Clara, CA, US), according to the manufacturer’s instructions. After 17 hours hybridization, slides were washed in staining dishes (Cat.# 121, Thermo Shandon, Waltham, MA, US) with Gene Expression Wash Buffer Kit(Cat.# 5188-5327, Agilent technologies, Santa Clara, CA, US), followed the manufacturer’s instructions

-        Blocking buffer: 10X Blocking Agent (Agilent p/n 5188-5281) diluted to 1X by nuclease-free water

-       Hybridization Buffer: 2x GEx Hybridization Buffer HI-RPM

-       wash buffer: 0.005% Triton X-102 in the Gene Expression wash buffers

Part 5 Measurements

-         Slides were scanned by Agilent Microarray Scanner (Cat#G2565CA, Agilent technologies, Santa Clara, CA, US) with default settings, Dye channel: Green, Scan resolution=3μm, PMT 100%, 20bit. Data were extracted with Feature Extraction software 10.7 (Agilent technologies, Santa Clara, CA, US). Raw data were normalized by Quantile algorithm, limma packages in R.

-         There are several repeat quality control probes in microarray to calculate CV value to assess the stability of microarray.

Part 6 Normalization controls

-        The original scanned chip data is normalized by the limma package in R. The algorithm used is Quantile, and the normalized signal value is the signal value calculated by log2.

-        When comparing data across a set of one-color microarrays, a simple linear scaling of the data is usually sufficient for most experimental applications. As Agilent determined, use the 75th percentile signal value of all of non-control probes to normalize Agilent one-color microarray signals for inter-array comparisons.

Figure 2  FaDu/DDP4 - miRNA Microarray

Figure 1  FaDu - miRNA Microarray

Figure 3  FaDu - ceRNA Microarray

Figure 4  FaDu/DDP4 - ceRNA Microarray